March 10, 2021

Communications Security, Reliability, and Interoperability Council VII

REPORT ON

RECOMMENDATIONS FOR IDENTIFYING OPTIONAL SECURITY FEATURES THAT CAN DIMINISH THE EFFECTIVENESS OF 5G SECURITY

Working Group 3: Managing Security Risk in Emerging 5G Implementations

Table of Contents

1	Results in Brief	4

1.1	Executive Summary	4

2	Introduction	4

2.1	CSRIC VII Structure	5

2.2	Working Group 3 Team Members	5

3	References	6

4	Objective, Scope, and Methodology	7

4.1	Objective	7

4.2	Scope	7

4.3	Methodology	8

5	Background and Related Security Activities	9

5.1	3GPP General Architecture	9

5.2	Overview of 3GPP Security Architecture	12

5.3	5G Security Feature Enhancements	14

5.3.1	Increased Home Control	14

5.3.2	Unified Authentication Framework	14

5.3.3	Security Edge Protection Proxy (SEPP)	14

5.3.4	Security Anchor Function (SEAF) and Authentication Framework	15

5.3.5	Serving Network Authentication	15

5.3.6	Mitigation of Bidding Down Attacks	16

5.3.7	Subscriber Identifier Privacy	16

5.3.8	User Equipment (UE) Configured Radio Restriction	16

5.4	5G Capabilities that are Mandatory to Implement but Optional to Deploy	17

6	Analysis and Recommendations	24

6.1	Analysis	24

6.1.1	User plane confidentiality	24

6.1.2	User plane integrity	24

6.1.3	Authentication	24

6.1.4	Authorization for Network Services within a PLMN	27

6.1.5	Integrated Access Backhaul (IAB)	28

6.1.6	Network Slice-Specific Authentication and Authorization (NSSAA)	29

6.1.7	Client Credentials Assertion (CCA)	29

6.2	Recommendations	30

6.2.1	Recommendations to the FCC	30

6.2.2	Recommendations to Industry	32

Annex A: Examples of Other Findings and Limitations	34

Annex B: Glossary of Terms	36

Table of Figures

Figure 4.1 - Scope of WG3 (Option 2)	8

Figure 4.2 - NIST SP 800-39 Managing Information Security Risk, Risk Management Process	9

Figure 5.1 - 5G Core Network Evolution	11

Figure 5.2 – A Few of the Core Network Options Considered in 3GPP	12

Figure 5.3 - The 3GPP 5G Security Architecture SOURCE: 3GPP	13

Figure 5.4 - The role of the SEPP in the security architecture. SOURCE: 3GPP	15

Figure 6.1 - Mutual Authentication between NFs via SCP using indirect communications	26

Figure 6.2 - Service Access Authorization using OAuth Tokens	27

Table of Tables

Table 1 - Working Group Structure	5

Table 2 - List of Working Group Members	6

Table 3 - List of Working Group Alternate Members	6

Table 4 - 3GPP Enhanced Security Features	17

# Results in Brief

## Executive Summary

5G is the fifth-generation mobile network that delivers higher multi-Gbps peak data speeds, ultra-low latency, more reliability, massive network capacity, increased availability, and a more uniform user experience to more users. Higher performance and improved efficiency empower new user experiences and connects new industries (often referred to as “verticals”); Examples of which include connected vehicles, smart cities, industrial automation, eHealth, Internet of Things (IoT), and many others.

The Federal Communications Commission (FCC) tasked CSRIC VII, via Working Group 3 (WG3), to identify optional security features in proposed or work-in-progress 5G standards that, without implementation, can diminish their effectiveness.

In developing 5G specifications the 3GPP group has addressed many of the security and privacy threats found in 4G LTE. This Report examines the optional security features of 5G New Radio (NR) and the new 5G Core network (5GC), with a primary focus on what is commonly referred to as the Standalone (SA) architecture.

The report focuses on the optional security features for networks deployed in the United States; however, there could be scenarios where the network of a U.S. carrier interfaces with a foreign network carrier that may not deploy the same optional security features (e.g., roaming). Carriers should be able to draw upon their experience in 4G LTE in handling these scenarios.

The report begins with a high-level background and related security activities in 3GPP. Next, an analysis of key optional security features of 5G is performed including a comparison of 3GPP Releases 15 and 16. Finally, the report concludes with several recommendations on how to mitigate potential 5G security threats, as well as proposed future work.

# Introduction

5G wireless and network technology is enabling a new wave of innovation that will impact many aspects of people’s lives from connected vehicles to healthcare and internet of things. To meet this need, not only is it critical that 5G networks are highly capable and reliable, it is essential that they are highly secure, ensuring the confidentiality and integrity of their intended use.

CSRIC VII [Ref 7] examined the 5G NR and 5G 5GC defined by 3GPP, and provided a summary of the new architecture, features, and capabilities of the systems defined by 3GPP in a report.  That report explored the security features defined as part of this new specification and analyzed the threats these features were designed to address.  Additionally, as the new 5G systems are expected to be deployed using so called “cloud like” capabilities, that earlier report also identified many Information Technology (IT) security developments that may enhance the deployments of any 5G network and particular those that incorporate “cloud like” infrastructure.

One aspect not addressed in that earlier report was regarding the flexible and optional security services defined by 3GPP.  As 3GPP is a global specification development organization, many of the features and functions described have a degree of flexibility or optionality defined to accommodate many different regional requirements, and to accommodate the different regulatory authorities where these technologies may be used.  In this report the security features of 3GPP Release 15 and 16 are analyzed further to examine any flexible or optional capabilities, to identify any common operational best practices for these capabilities and where necessary identify any areas of potential regulation. The primary focus of this report is 3GPP Release 16.

## CSRIC VII Structure

CSRIC VII was established at the direction of the Chairman of the FCC in accordance with the provisions of the Federal Advisory Committee Act, 5 U.S.C. App. 2.  The purpose of CSRIC VII is to provide recommendations to the FCC regarding ways the FCC can strive for security, reliability, and interoperability of communications systems.  CSRIC VII’s recommendations will focus on a range of public safety and homeland security-related communications matters.  The FCC created informal subcommittees under CSRIC VII, known as Working Groups (WG), to address specific tasks.  These working groups must report their activities and recommendations to the Council as a whole, and the Council may only report these recommendations, as modified or ratified, as a whole, to the Chairman of the FCC.

| Communications Security, Reliability, and Interoperability Council (CSRIC) VII |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CSRIC VII Working Groups |  |  |  |  |  |
| Working Group 1: Alert Originator Standard Operating Procedures & Duplicate NWS Alert | Working Group 2: Managing Security Risk in the Transition to 5G | Working Group 3: Managing Security Risk in Emerging 5G Implementations | Working Group 4: 911 Security Vulnerabilities during the IP Transition | Working Group 5: Improving Broadcast Resiliency | Working Group 6: SIP Security Vulnerabilities |
| Chair: Craig Fugate, APTS/ CoChairs: Michelle Mainelli, NWS Terri Brooks, T-Mobile | Chair: Kathy Whitbeck, Nsight | Chair: Farrokh Khatibi, Qualcomm | Chair: Mary Boyd, West Safety Services | Chair: Pat Roberts, Florida Association of Broadcasters | Chair: Danny McPherson, Verisign |
| FCC Liaison: James Wiley/ David Munson | FCC Liaison: Kurian Jacob | FCC Liaison: Steven Carpenter | FCC Liaison: Rasoul Safavian | FCC Liaison: Robert “Beau” Finley | FCC Liaison: Ahmed Lahjouji |

Table 1 - Working Group Structure

## Working Group 3 Team Members

Working Group 3 consists of the members listed below.

| Name | Company |
| --- | --- |
| Farrokh Khatibi (Chair)* | Qualcomm Technologies, Inc. |
| Billy Bob Brown, Jr | CISA DHS |
| Brian K. Daly* | AT&T Services Inc. |
| Christopher (Chris) Joul | T-Mobile |
| Mohammad Khaled | Nokia Bell Labs |
| Chandra Kotaru* | AWARN Alliance |
| Michael Liljenstam | Ericsson |
| John Marinho | CTIA |
| Danny McPherson* | Verisign |
| Susan M. Miller* | ATIS |
| Krisztina Pusok* | American Consumer Institute |
| Travis Russell* | Oracle Communications |
| D.J. Shyy | MITRE |
| Kathy Whitbeck* | Nsight |
| Brian Trosper* | Verizon |
| Steve Watkins* | Cox Communications |
| Jeffrey Wirtzfeld | Lumen |
| Fei Yang | Comtech |
| FCC Liaison, Steven Carpenter |  |

Table 2 - List of Working Group Members

* CSRIC VII Members

The Working Group members had an option to nominate an alternate to participate in the discussions when they were unavailable. Although these alternates are not a member of the Working Group and may not vote, they provided valuable input towards the completion of this report that should be acknowledged. Working Group 3 alternate members are listed in

Table 3.

| Name | Company |
| --- | --- |
| Steve Barclay | Alliance for Telecom Industry Solutions (ATIS) |
| Vinod Choyi | Verizon |
| Martin Dolly | AT&T Services Inc. |
| Yong Kim | Verisign |
| Andrew Schnese | Nsight |
| Greg Schumacher. | T-Mobile |

Table 3 - List of Working Group Alternate Members

# References

- 3GPP TS 23.501, “System architecture for the 5G System (5GS)”.

- 3GPP TS 22.011, “Service accessibility”.

- 3GPP TS 33.501, “Security architecture and procedures for 5G system”.

- 3GPP TS 33.310, “Network Domain Security (NDS); Authentication Framework (AF)”.

- 3GPP TS 33.210, “Network Domain Security (NDS); IP network layer security”

- CSRIC VI Report on Recommendations to Mitigate Security Risks for Diameter Networks (March 14th, 2018).

- CSRIC VII Report on Risks Introduced by 3GPP Releases 15 and 16 5G Standards (September 16, 2020).

# Objective, Scope, and Methodology

## Objective

The FCC directed CSRIC VII  to review risks to 5G wireless technologies that can lead to the loss of confidentiality, integrity, and availability of wireless network devices.  CSRIC VII will recommend best practices to mitigate the risks for each vulnerability it identifies and address recently proposed solutions by security researchers.

Additionally, the FCC directed CSRIC VII to recommend any updates, if appropriate, to the 3GPP SA3 (security working group) standards, including digital certificates and pre-provisioned Certificate Authorities, to mitigate these risks and then place the vulnerabilities on a scale that accounts for both risk level and remediation expense.

Finally, the FCC directed CSRIC VII to identify optional features in 3GPP standards that can impact the effectiveness of 5G security and make recommendations to address these gaps.

To ensure comprehensive analysis of the above requirements two approaches have been taken by CSRIC VII. The first report [Ref 7] took a top-down approach, looking first at risks, then progressing to enhancements and options in 3GPP specifications. This report takes a bottoms-up approach first looking at feature optionality and from there identifying any recommendations to address perceived risks to the standalone architecture.

## Scope

The scope of this report is to identify optional security features in 3GPP standards that, if not implemented, can diminish the effectiveness of 5G security, and make recommendations to address these gaps. The analysis and assessment are based upon industry best practices and standards including the National Institute of Standards and Technology (NIST) and International Organization for Standardization (ISO) Standards.

The optional security features analyzed in this report are mainly from 3GPP TS 33.501 [Ref 3] which is the primary security standard for Standalone 5G.  A few other optional security features defined in other 3GPP specifications are also covered in this report.

Due to the time allotted for this report and the scale of the full 5G specifications a comprehensive review was not possible.  Optional security feature categories not covered include:

- Security of capabilities still being developed in 3GPP such as interworking between 5G SA and 4G networks, and

- Existing optional security capabilities which were brought forward to 5G from 4G and earlier such as Network Domain Security and IP Multimedia Subsystem (IMS) Security.

Within the set of optional security features analyzed in this report, only those with options to deploy or not deploy were considered.  Optional security features where the feature is mandatory to deploy with a choice between several defined approaches were not analyzed in this report.

Although CSRIC VII reviewed the security of all architectural options identified in 3GPP specifications, the primary focus is the options with the 5G Core (5GC), especially the Standalone architecture of option 2 (see Figure 4.1) and 3GPP TS 33.501 [Ref 3]. Note that, options 4 and 7 have also been discussed in 3GPP, but option 2 is the primary architecture that will be used by U.S. carriers.

Out of Scope

CSRIC VII WG3

CSRIC VII WG2

Figure 4.1 - Scope of WG3 (Option 2)

## Methodology

CSRIC VII was asked to examine the security risks associated with 5G, as well as the optional security features in 3GPP specifications, that result from both infrastructure and device capabilities that may introduce incremental security risk. The 3GPP, as well as several other standards organizations, continue work on 5G standards.

The methodology used in this Report is based on NIST SP 800-39 as shown in Figure 4.2. In order to frame the 5G security risks, risk context must be framed, the corresponding risks must be assessed, risk response recommendations must be identified, and ongoing monitoring must be performed.

Figure 4.2 - NIST SP 800-39 Managing Information Security Risk, Risk Management Process

# Background and Related Security Activities

The 3GPP standards process is an ongoing effort that will continue to release updates for adoption. This specification defines the security architecture (i.e., the security features and the security mechanisms for the 5G system and the 5G core) and the security procedures performed within the 5G system including the 5G core and the 5G NR. The main features are defined in 3GPP TS 33.501 [Ref 3].

This section provides some background and related security activities. Additional material is available in CSRIC VII WG3 Report [Ref 7].

## 3GPP General Architecture

The 5G Core Network (5GC) evolution from the 4G Core architecture is shown in Figure 5.1. The evolution shows the migration from the traditional 4G point-to-point configuration to an architecture that is based on functional interconnections through a “common bus” approach that permits functional elements to communicate based on service definition. The architecture is called a Service Based Architecture (SBA), where key differentiators include:

- Service Based Architecture (SBA)

  - Direct and Indirect communication and delegated discovery through Service Communication Proxy.

  - Introduction of Network Functions (NF) Set and NF Service sets – For 5GC the control plane functionality and common data repositories of a 5G network are delivered by way of a set of interconnected Network Functions, each with authorization to access each other’s services or sets of services.

  - Selection and reselection within NF Set – The 5GC employs a centralized discovery selection framework that leverages a Network Repository Function (NRF). The NRF maintains a record of available NF instances and their supported services. It allows other NF instances to subscribe and be notified of registrations from NF instances of a given type. The NRF supports service discovery, by receipt of Discovery Requests from NF instances and details which NF instances support specific services.

  - Convert IMS interfaces to utilize SBA – The 5GC provides the mechanism to convert today’s IP Multi-Media Sub-system (IMS) to use of a Services Based Architecture (SBA) that provides flexibility and scale in service delivery as well as support for new capabilities such as Network Slicing.

- Network Slicing - Network slicing is a fundamental new capability of 5GC that provides flexibility when deploying diverse network services and applications. A logical end-to-end network slice has pre-determined service capabilities, traffic characteristics and service level agreements and includes the virtualized resources required to service the needs of a group of subscribers, including a dedicated User Plane Function (UPF), Session Management Function (SMF) and Policy Control Function (PCF). Key capabilities include:

  - Slice specific authentication and authorization

  - Improvements in slice interworking with Evolved Packet Core (EPC)

- Control Plane/User Plane (CP/UP) Split- 5G standards require the separation of the Control and User Planes (traffic paths within the 5G architecture) within the network. The separation is driven by the notion that user-plane and control plane functionalities are quite different given the performance characteristics of exchanging signaling messages between network control functions and the needs of a transport network that carries user/application traffic.

- Integration of cloud/edge computing – The integration of cloud and edge computing is another new fundamental capability in 5G. The architecture no longer relies on centralized, cloud-based computing, and instead utilizes cloud capabilities at the edge of the network to deliver real-time performance and reduced latency.

- Flow based Quality of Service (QoS) – QoS in 5G is flow based, rather than bearer based as in 4G. Packets are classified and marked using QoS Flow Identifier (QFI). The 5G QoS flows are mapped in the RAN to Data Radio Bearers (DRBs) unlike 4G LTE where mapping is one-to-one between EPC bearers and LTE radio bearers.

Figure 5.1 - 5G Core Network Evolution

Figure 5.2 illustrates a few of the options considered in 3GPP.

Figure 5.2 – A Few of the Core Network Options Considered in 3GPP

As described in Figure 5.2, in previous networks, network appliances connect to one another using point-to-point interfaces. In voice services, there are specific network functions that are used for every voice call. Likewise, in data sessions there are specific network appliances that support every data session. Point to point connections and specific network appliances proves problematic for Internet of Things (IoT), where support of millions or even billions of devices trying to send simple data sets to an application do not need all of the functionality provided in traditional networks. This is one of the drivers to moving to SBA.

SBA has been around for some time, and many Web applications use SBA to deliver services. This concept has been adopted by the 3GPP and used in 5G where network functions are delivered as services, as needed.

The 5G voice architecture is IP based and uses the IP Multimedia Subsystem (IMS) in the same way 4G LTE IP voice uses the IMS. A fallback capability exists when the IMS is not available or critical capabilities are not available on NR/5GC. In this case these IP voice calls may use NR or LTE via the 4G EPC to access the IMS. Security capabilities when using LTE and the 4G EPC will still have the same limitations as defined in 3GPP TS 33.401 and documented in previous CSRIC reports.

Examples of critical capabilities that may not be available upon initial 5GC deployments are those required for support of priority services, whether civil 9-1-1 or Government Emergency Telecommunications and Wireless Priority Services (GETS/WPS). These services will migrate to the 5GC when these critical capabilities are developed and deployed.

## Overview of 3GPP Security Architecture

The 3GPP security architecture from 3GPP TS 33.501 [Ref 3] is shown in Figure 5.3 below.

Rather than depicting each functional element of the network and the interfaces between these elements as depicted in Figure 5.1, this security architecture describes several security domains where elements supporting related functions are grouped into a security domain. The 3GPP security approach applies common security requirements apropos to the purpose of the security domain as well as applies security approaches targeting the domain's security risks.

Figure 5.3 - The 3GPP 5G Security Architecture SOURCE: 3GPP

ME: Mobile Equipment

SN: Serving Network (separate from the Home Environment in the case of roaming)

HE: Home Environment

USIM: Universal Subscriber Identity Module

AN: Access Network

Network access security (I): the set of security features that:

- enables user's mobile equipment to authenticate the network and access services via the network securely, including the 3GPP access and non-3GPP access. In particular, to protect against attacks on the (radio) interfaces.

- enables the network to authenticate the user (via the USIM) and mobile equipment accessing the network.

- provides security context delivery from the serving network to user equipment for the access security.

Network domain security (II): the set of security features that enable network nodes to securely exchange signaling data and user plane data.

User domain security (III): the set of security features that secure the user access to mobile equipment.

Application domain security (IV): the set of security features that enable applications in the user domain and in the provider domain to exchange messages securely.

SBA domain security (V): the set of security features that enables network functions of the SBA architecture to securely communicate within the serving network domain and with other network domains (for roaming). This includes the network element registration, discovery, authorization security aspects, and the protection for the service-based interfaces.

Visibility and configurability of security (VI): the set of features that enable the user to be informed whether a security feature is in operation or not.

## 5G Security Feature Enhancements

### Increased Home Control

Home control is used for authentication of the device location when the device is roaming. It allows the home network to verify the device is actually in the serving network when the home network receives a request from a visited network.

This was added to address the vulnerabilities found in 3G and 4G networks where networks could be spoofed and send false signaling messages to the home network in an effort to request the International Mobile Subscriber Identity (IMSI) and location of a device. This information could then be used to intercept voice calls and text messages.

### Unified Authentication Framework

In 5G networks, authentication will be access agnostic. The same authentication methods are used for both 3GPP and non-3GPP access networks (5G radio access and Wi-Fi access, for example).

Native support of Extensible Authentication Protocol (EAP) is key. This allows for new plug-in authentication methods to be added in the future without impacting the serving networks.

### Security Edge Protection Proxy (SEPP)

To protect messages that are sent over the N32 interface (between two 5G networks), the SEPP is defined as the entity sitting at the perimeter of the 5G network. The SEPP:

- Receives all service layer messages from the network function and protects them before sending them out of the network on the N32 interface and

- Receives all messages on the N32 interface and forwards them to the appropriate network function after verifying security, where present.

The SEPP implements application layer security for all the information exchanged between two network functions across two different Public Land Mobile Networks (PLMNs).

Figure 5.4 - The role of the SEPP in the security architecture. SOURCE: 3GPP

Protection for the Hypertext Transfer Protocol (HTTP) message payload, sensitive information contained in the HTTP message header, and the Request Uniform Resource Identifier (URI) is provided by the SEPP. However, not all information in the payload gets the same level of protection. Some information may require end-to-end encryption, while other information may only require integrity protection end-to-end, while still allowing modification of the message by intermediary Internetwork Packet Exchange (IPX) providers.

The SEPP provides confidentiality of sensitive information as it is passed between two networks. This prevents intermediary networks (such as IPX) from being able to see this sensitive information (such as authentication vectors).

If there are parameters in the data that need modification by the IPX (such as parameters modified by mediation for interoperability), both networks must agree on what changes can be made and those changes must be integrity protected. When modifications are made by an IPX, the receiving network verifies the changes. The SEPP can detect any unauthorized message modifications received by an IPX.

The SEPP also provides traditional gateway functions, such as topology hiding, protection from malformed packets, protection against signaling storms, and validation of the message source.

In Release 16 the principle of Inter-PLMN Security for User Plane traffic in roaming scenarios has begun to be addressed by 3GPP – The role of the UPF is expanded to include protection for the N9 traffic between two roaming PLMNs.  As is already defined in 5G architecture, the UPF can be segmented according to function, role, or deployment scenarios – thus this new functionality can be implemented either in a combined node with other UPF functionality, or in a dedicated inter-PLMN UPF at the edge of the network.

### Security Anchor Function (SEAF) and Authentication Framework

5G introduces the concept of an anchor key, with the new function of the SEAF. The SEAF is co-located with the Access and Mobility Management Function (AMF) as per Release 15 of the 5G network specifications. The SEAF allows for the re-authentication of the device when it moves between different access networks, or even in serving networks without having to run the full authentication method (e.g., Authentication and Key Agreement (AKA) authentication). This reduces the signaling load on the home network during various mobility services.

### Serving Network Authentication

Binding authentication keys to a serving network prevents network spoofing. The serving network thereby authenticates itself to the user equipment. The serving network authentication is used for selected access technologies (e.g., 5G networks and Wi-Fi access).

### Mitigation of Bidding Down Attacks

5G includes requirements that prevent an attacker from attempting a bidding down attack that makes the user equipment and the network entities believe that the other side does not support a security feature, even when both sides in fact support that security feature.

This prevents fake base stations from forcing devices with earlier access technologies (e.g., 2G, 3G) to avoid more advanced security features developed in later releases.

### Subscriber Identifier Privacy

Each subscriber in the 5G system is assigned a globally unique Subscriber Permanent Identifier (SUPI). The SUPI is provisioned in the Unified Data Management (UDM)/User Data Repository (UDR) and is only used within the 5G network. The SUPI incorporates the IMSI as part of its value, allowing the IMSI to be extracted and used when interworking with 3G/4G networks. The SUPI also incorporates the Mobile Country Code (MCC)/Mobile Network Code (MNC) for identifying the home network when used in 5G.

The SUPI replaces the IMSI used in previous networks, but the SUPI is never disclosed over the air in the clear when a mobile device is establishing a connection, prior to authentication.

To avoid disclosing the SUPI, a Subscription Concealed Identifier (SUCI) is used until the device and network are authenticated. Only then is the SUPI disclosed by the home network to the serving network but never sent in the clear over the air. The MCC/MNC portion of the SUPI is not protected, allowing other networks to quickly determine the home network in the SUPI.

### User Equipment (UE) Configured Radio Restriction

3GPP SA1 developed requirements in 3GPP TS 22.011 [Ref 2] for the UE to be able to enable and disable (restrict) usage of the radio technologies available on the UE.

This UE feature allows the user or home network to configure the available radio technologies which can be used by the UE. The radio technologies on which all UE operation can be restricted is: 4G/LTE, GSM/EDGE, WCDMA, or 5G NR. The configuration settings persist between power cycles and reboots.

When a radio technology is restricted and if a prioritized service (such as emergency services, Multimedia Priority Service (MPS), or mission critical services) is needed, and no other networks are available on the usable radio technologies, the prioritized service can override the radio technology restrictions.

3GPP has not begun the stage 2 and 3 development needed for implementation of this capability.  As this work is not scheduled as of the publication of this report, the earliest possible completion is Release 17.

## 5G Capabilities that are Mandatory to Implement but Optional to Deploy

In this section we compare the optional security features of 5G in Releases 15 and 16. Table 4 lists categories of enhanced security features and compares requirements in terms of mandatory/optional use across Release 15 and 16.  Table 4 uses the terms “mandatory” and “optional” according to their use in 3GPP. “Mandatory” means that a device, node, or function shall support the feature, however 3GPP makes no requirement that the feature or function be used (i.e., deployed) to allow for individual networks to accommodate local requirements and regulations.  Furthermore, the use of these terms in Table 4 does not imply any recommendation by CSRIC VII, all such recommendations are contained in section 6.2 of this report.

Regarding mandatory table entries, it is worth noting that across several categories (including  NAS signaling integrity protection and Radio Resource Control (RRC) signaling integrity protection), FCC regulations require completion of emergency (9-1-1) calls from unauthenticated devices. Thus, unlike other traffic, unauthenticated emergency calls use a null algorithm and cannot establish a security context.

Table 4 - 3GPP Enhanced Security Features

| Security Category | 5G in R-15 (3GPP TS 33.501) | 5G in R-16 (3GPP TS 33.501) |
| --- | --- | --- |
| NASI signaling confidentiality | Optional | Same |
| NAS signaling integrity | Mandatory | Same |
| User plane confidentiality | Optional | Same |
| User plane integrity | Optional | Same |
| RRCii signaling confidentiality | Optional | Same |
| RRC signaling integrity | Mandatory | Same |
| SUPI/IMSI privacy | SUPIIII encryption is optional (has exceptions) | Same |
| Primary Subscriber Authentication (5G AKA / EAP-AKA') | Has optional features | Same |
| Network slice specific authentication and authorization function (NSSAAF) | Does not exist | Optional |
| Certificates – gNB enrollment | Optional IV | Same |
| Certificates – IPSec 10 | Mandatory (with some optional implementation mechanisms V) | Same |
| Certificates – TLS 10 | Mandatory (with some optional implementation mechanisms VI) | Same |
| Network security – IPSec | Optional | Same |
| Core network security – TLS | Optional | Same |
| Security for SBI interfaces: TLS protection for direct connection between NF and SEPP | Mandatory | Mandatory |
| Security for SBI interfaces 11: Protection between SEPPs | Mandatory | Mandatory 12 |
| Security for SBI interfaces 11: Authentication and authorization between network functions and the NRF | Mandatory | Mandatory 12 |
| Security for SBI interfaces 11: Authentication and authorization between network functions | Does not exist | Mandatory 12 |
| Security for SBI interfaces : Authentication between SEPP and SCP | Does not exist | Mandatory 12 |
| Security for SBI interfaces 11 15: Authentication and authorization between SCP and network functions | Does not exist | Mandatory 12 |
| Security for SBI interfaces 11 15: Authentication and authorization between SCPs | Does not exist | Mandatory 12 |
| Security for SBI interfaces 11: Client credentials assertion and authentication | Does not exist | Optional |
| Security for non-SBA interfaces internal to the 5GC and between PLMNs | Mandatory with optional implementation mechanism | Same |
| Roaming Security – Security Edge Protection Proxy (SEPP) | Mandatory | Same |
| Roaming Security – Inter PLMN Encryption (inter-PLMN UP security (IPUPS) | Does not exist | Mandatory 12 |
| Key size | 256 bits allowed | Same |
| IAB security | Does not exist | Optional 19 |
| Residential Gateway security | Does not exist | Mandatory 12 for RG to implement clause 5.2 but clause 5.2 has Optional features 20 |
| Standalone NPN | Does not exist | Mandatory for standalone NPN |
| Security for the interface between Wireline W-5GAN and 5GC | Does not exist | Mandatory 12 23 |

Note 1: In this document, AS is the same as RRC + User Plane.

I: NAS: non access stratum = control plane between UE and the core network

II: RRC: radio resource control = control plane between UE and the base station

III: SUPI: subscription permanent identifier

The following are the exception scenarios when the subscription concealed identifier (SUCI) can use null scheme.

- The UE is requesting emergency services

- The home network does not have a public key

- The UE is configured by the home network to use the null scheme

- The ME (rather than the USIM) is configured to compute SUCI and there is no prioritized list of protection schemes on the USIM

IV: In TS33.501, it has the following: “The certificate enrolment mechanism specified in TS 33.310 (Network Domain Security [NDS]; Authentication Framework [AF]) for base station should be supported for gNBs. The decision on whether to use the enrolment mechanism is left to operators.”

V: In 3GPP TS 33.501 [Ref 3], it has the following: “In order to protect the N2 reference point, it is required to implement IPsec ESP and IKEv2 certificates-based authentication as specified in sub-clause 9.1.2 of the present document. IPsec is mandatory to implement on the gNB and the ng-eNB. On the core network side, a security gateway (SEG) may be used to terminate the IPsec tunnel.” And “In order to protect the traffic on the N3 reference point, it is required to implement IPsec ESP and IKEv2 certificate-based authentication as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection. IPsec is mandatory to implement on the gNB and the ng-eNB. On the core network side, a SEG may be used to terminate the IPsec tunnel.”

VI: In 3GPP TS 33.501 [Ref 3], it has the following : “Subscriber certificates that are used with EAP-TLS typically include static validity times. A certificate revocation list (CRL) as specified in RFC 5280 and online certificate status protocol (OCSP) as specified in RFC 6960 are means for the issuing certificate authority (CA) to revoke the certificates before their scheduled expiration date. In 5G security architecture, the UDM/ARPF is responsible for such subscriber status information. EAP-TLS peers and servers may also support Certificate Status Requests (OCSP stapling) as specified in RFC6066 which allows peers to request the server's copy of the current status of certificates.”

In addition to the security requirements for 5G contained in 3GPP TS 33.501 [Ref 3], 3GPP has defined several Security Assurance Specifications (SCAS) for various network functions of the 5G system. These SCAS define the security tests to be performed by independent security test labs as part of the GSMA Network Equipment Security Assurance Scheme (NESAS). In Release 15 and 16 the SCAS documents cover the following 3GPP 5G network functions:

- 3GPP TS 33.511: Next Generation Node B (gNB)

- 3GPP TS 33.512: Access and mobility management function (AMF)

- 3GPP TS 33.513: User plane function (UPF)

- 3GPP TS 33.514: Unified data management (UDM) network product

- 3GPP TS 33.515: Session management function (SMF) network product

- 3GPP TS 33.516: Authentication server function (AUSF) network product

- 3GPP TS 33.517: Security edge protection proxy (SEPP) network product

- 3GPP TS 33.518: Network repository function (NRF) network product

- 3GPP TS 33.519: Network exposure function (NEF) network class

Also note there are ongoing studies and work in 3GPP Release 17 to further define and expand on the SCAS.

# Analysis and Recommendations

## Analysis

### User plane confidentiality

Confidentiality protection for the user plane (UP) is applied at the packet data convergence protocol (PDCP) layer, and no layers below PDCP are confidentiality protected. Confidentiality protection for non-access stratum (NAS) is provided by the NAS protocol. User data sent via the user data repository (UDR) may be confidentiality protected.

### User plane integrity

4G LTE supports integrity protection of over the air control plane (signaling) traffic exchanged between the UE and the RAN but not the user plane traffic. 5G requires mandatory support of integrity protection of the user plane traffic. This feature mitigates a sophisticated man-in-the middle attacker from tampering with (or modifying) any sensitive over the air user plane data that is not protected (e.g., unprotected DNS messages).

Although the user plane integrity protection at the full data rate of UE was supported in Release 15, it was dependent on the capability of the UE (e.g., whether it has the necessary hardware capability). The Release 15 UEs were required to support integrity protection only at the rate of 64kbps.

In Release 16, all UEs are mandated to support the user plane integrity protection at the full data rate.

### Authentication

#### Primary Authentication (Subscription Authentication)

Primary authentication enables mutual authentication of the UE / subscription with the home network. 3GPP TS 33.501 specifies that a UE and the relevant network functions (e.g., AMF, AUSF) support both 5G authentication and key agreement (AKA) as well as the improved extensible authentication protocol method (EAP-AKA’) procedures. The 5G AKA unlike the 4G Evolved Packet System (EPS) AKA enables mutual authentication between the UE and the home network which is comparable to the EAP-AKA’ procedure. An operator shall select either one of the authentication procedures for subscription authentication of the UE.

After the completion of the primary authentication, a 5G security context is established between the UE and the network. The 5G security context consists of the Access Stratum (AS) security context and a Non-Access Stratum (NAS) security context. Implicit authentication occurs between the UE and the AUSF, AMF and the gNB, wherein control plane messages are authenticated by means of key possession.

Optionally, additional secondary or slice-specific authentications may be utilized after a successful primary authentication based on operator or enterprise customer service needs. The use of an authentication, authorization, accounting (AAA) server for this additional authentication provides support for slice access control dedicated to specific services or enterprises. An enterprise can integrate this secondary slice access control into its existing AAA used for internal network access control.

#### Authentication between Network Components

All communications between OA&M systems that are used for configuring the various network components including RAN and Core functions shall be authenticated and protected so that un-authorized modification of settings and software configurations via local or remote access can be mitigated. Mechanisms for this authentication/protection are described in 3GPP TS 33.310 [Ref 4].

##### Authentication between 5G Core components

3GPP TS 33.501 provides the ability for core Network Functions (NF) that communicate with one another using Service-Based Interfaces (SBI) to be mutually authenticated. NF-NF communications may be performed by direct or indirect means (via SCP). In the case of direct communications and where service-based interfaces are used the NFs mutually authenticate each other using X.509v3 certificates. The communications are protected for integrity, replay and confidentiality protection using mutual TLS 1.2 or TLS 1.3. Mutual authentication between the NFs as specified in 3GPP TS 33.310 [Ref 4] can be carried out using X.509v3 certificates that have been issued by a certificate authority and performs the role of trust anchor to all the NFs within a PLMN’s 5G core network.

In the case of indirect communications, an NF consumer sets-up mutual TLS connection with the SCP, where the NF consumer and SCP mutually authenticate one another using the provisioned X.509v3 certificates and a similar process is carried out between the SCP and the NF producer to setup a separate TLS connection. An example message flow from 3GPP TS 33.310 [Ref 4] is depicted in Figure 6.1.

Figure 6.1 - Mutual Authentication between NFs via SCP using indirect communications

Within the 5G core, some implementations may use virtualized platforms that do not have physically separate compute and store; in these deployments NFs should use certificates (e.g., TLS 1.2 or TLS 1.3) or other methods of mutual authentication.

Authentication based on X.509v3 certificates shall also carried out for NF-NF communications that are not based on SBI (e.g., PFCP based on UDP between SMF and UPF) and uses DTLS as the protocol of choice or where IPSec is used via a security gateway for NFs that do not natively support TLS.

##### Authentication between 5GC and 5G RAN components

The N2 interface between the AMF and the gNB or the CU carries the KgNB that is used for AS security context. It is also highly likely that the AMF and gNBs may not be co-located within the same data center and therefore the likelihood of tampering or eavesdropping on the interface that traverses across administrative domains is high. The KgNB is carried over GTP messages and therefore shall be protected using DTLS or IPSec, where a security gateway may be used on the core network side. Each gNB / CU shall be provisioned with X.509v3 certificates which is used for mutual authentication with an AMF.

##### Authentication between 5G RAN components

Depending on the deployment model of the various 5G RAN components (RU, DU and CU) authentication may be needed between the discreet components according to local conditions. Use of IPsec, for the F1-C interface, or alternatively DTLS should be used for performing authentication by means of X.509v3 certificates and provide integrity, replay and confidentiality protection.

### Authorization for Network Services within a PLMN

3GPP TS 33.501 [Ref 3] specifies two primary mechanisms for authorization of NF services:

- Static authorization

- Authorization by means of access token (OAuth token)

#### Static Authorization

A network repository function (NRF) performs a lookup of local authorization policies in order to provide an NF-C with the required services (e.g., discovery). Similarly, an NF-P is pre-configured with authorization policies in order that a requesting NF-C is provided with the appropriate services based on local policies that have been pre-provisioned at the NF-P.

#### Authorization by means of Access Token

Unlike static authorization, the NF services offered by NF-P are authorized by an authorization server, which is a service offered by the NRF. The NRF therefore performs the role of centralized authorization server. An NF-P offering services would register its services with the NRF. The authorization server uses the OAuth 2.0 framework as specified in IETF RFC 6749, where the grants are of the type Client Credentials Grant. An example message flow of authorization by means of access token is depicted in Figure 6.2.

Figure 6.2 - Service Access Authorization using OAuth Tokens

An NF-C initiates and performs mutual authentication by means of TLS with the NRF in order that it can perform discovery of services offered by NF-P in a secure manner. Based on the discovery results obtained from an NRF, the NF-C perform access token request for services offered by an NF-P or an NF-P type. It is highly recommended to protect registration, discovery and access token request via a mutually authenticated and secure TLS connection. The NRF based on the request determines if the request submitted by the NF-C is permitted based on the authorization rules and an access token with the appropriate claims is generated by the NRF. The access token that is generated shall be compliant to JSON Web Tokens as described in IETF RFC 7519 and are secured with digital signatures based on JSON Web Signature (JWS) as described in IETF RFC 7515.

The NF-C requesting services with an NF-P, submits the access token within an authorization header to the NF-P. The NF-P checks the authenticity of the token by verifying the digital signature (JWS) as that of the NRF, also verifies the validity (expiration time) of the token. The NF-P verifies that the audience claim in the access token matches its own identity (e.g., FQDN) or it’s NF-type. The NF-P also verifies that the identity of the “subject” field matches the NF-C’s TLS endpoint identity (e.g., FQDN). If the verification was successful, the NF-P executes the requested service to the NF-C.

In cases where indirect communications occur via SCP, Client Credentials Assertion (CCA) may be used, where the CCA is a token signed by the NF-C. It enables the NF-C to authenticate towards the NF-P or NRF by including the signed token in a service request. CCAs are expected to be more short-lived than NRF generated access tokens.

Further work on CCA is being done in 3GPP Release 17.

### Integrated Access Backhaul (IAB)

In Release 16, 3GPP introduced support to allow gNB to utilize the same frequencies used for user data to convey connectivity between distributed gNBs – this feature is called Integrated Access Backhaul (IAB).  IAB is an implementation choice by the network operator and thus the resulting security requirements are dependent on if this feature is used – if it is not used then the related security features are not applicable.  3GPP TS 33.501 [Ref 3] details the security requirements for IAB in annex M – these related to the secure environment for a distributed gNB (contained in clause 5.3.8) , securing the link (the F1 Interface) between the central and distributed units (described in clause 5.3.9), and authenticating & authorizing the IAB node using existing registration and authentication procedures (i.e., the IAB node has a SIM and some UE functionality).  Annex M also further details some of the IP security (IPSEC) parameters used to secure the F1 interface.  In addition, IAB allows for implementations to use either the F1 interface or user plane bearers for O&M traffic; in the case of IAB utilizing user plane bearers, confidentiality and integrity protection is required (as defined in clause 6.6 of 3GPP TS 23.501 [Ref 1]).

3GPP TS 23.501 [Ref 1] Section 5.3.9 includes a reference to DTLS being optional for mutual authentication of the IAB nodes utilizing the F1 interface, however mutual authentication in the IAB deployments requires the use of  SIM authentication (which provides mutual authentication), therefore the use of DTLS does not provide any additional protection to IAB deployments.

Since the optionality of security functionality is only related to the choice of implementing IAB or not, and the choice of implementing F1 or UP based O&M, no further analysis of IAB is needed.

### Network Slice-Specific Authentication and Authorization (NSSAA)

The Network Slice-Specific Authentication and Authorization (NSSAA) provides additional slice-specific authentication, authorization, re-authentication and revocation services in order that a UE is authorized to access a slice dedicated to specific services or enterprises. The NSSAA, an optional procedure, is used between a UE and a AAA server (AAA-S), which may belong to a third party (e.g., enterprise). The credentials used for the NSSAA procedure is separate from the 3GPP subscription credentials and is performed after the execution of a primary authentication procedure (e.g., 5G-AKA, EAP-AKA’). The EAP framework based on IETF RFC 3748 shall be used offering the flexibility to use various EAP methods. The AMF plays the role of an authenticator, while the NSSAAF is an intermediary that may perform protocol conversion (converting service-based messages to EAP) as needed and relays EAP messages towards a AAA-S. The AMF, based on a UE’s subscriber profile that allows access to S-NSSAI, triggers the NSSAA procedure (based on operator policy derived from service requested) via the Network Slice-Specific Authentication and Authorization Function (NSSAAF) in order that the UE is provided authorization to the particular dedicated slice, identified by the S-NSSAI. Protection of communications between the NSSAAF and the AMF, AAA-S / AAA-P (AAA proxy) respectively may be protected using TLS or NDS/IP as specified in 3GPP TS 33.501 [Ref 3].

### Client Credentials Assertion (CCA)

The Client Credentials Assertion (CCA) is a signed token generated by a NF-C in the indirect communications mode (via an SCP) that enables an NF-C to authenticate towards a receiving end point (NRF, NF-P) in an end-to-end manner within a PLMN. The CCA shall be compliant to JSON Web Token IETF RFC 7519 and shall include the NF instance id of the NF-C, timestamp (iat), expiration time (exp) and the NF-type of the audience (NRF or NF-P). The lifetime of a CCA is expected to be much shorter than an NRF generated access token. The CCA shall be digitally signed by the NF-C using its private key and compliant to JSON Web Signature IETF RFC 7515. The CCA and the signing certificate are sent by the NF-C as part of an access token request or a service request to an NRF or NF-P respectively. The receiving NF (NRF or NF-P) shall verify the claims and the digital signature in the CCA and shall ensure that the identity of the NF-C included in the CCA matches with the identity in the signing certificate. It should be noted that the CCA does not provide message integrity and authenticity to the entire request and nor can it be used for end-to-end authentication of responses from a NF-P to an NF-C.

## Recommendations

### Recommendations to the FCC

#### Previous CSRIC Recommendations

CSRIC VII commends the FCC’s efforts to support CSRIC recommendations as shown by previous Public Notices (PNs).  CSRIC VII recommends that the FCC encourage industry for continued implementation of CSRIC’s prior recommendations   and continue to promote awareness.

#### Potential future CSRIC Topics

As described in Section 4.2 (Scope) of this report, the work of this CSRIC was not exhaustive.  CSRIC VII therefore recommends that the FCC consider further CSRIC work to expand the security analysis coverage of 5G SA.

CSRIC VII recommends future CSRICs consider:

- Security of capabilities still being developed in 3GPP future Releases 17 and 18, such as interworking between 5G SA and 4G networks.

- Existing optional security capabilities which were brought forward to 5G from 4G which have not been addressed in previous CSRICs or warrant revisiting for 5G SA.  Examples include Network Domain Security and IMS Security

- Mandatory security features to deploy with a choice between several defined approaches can be analyzed for which choice may be recommended. Some examples are in the following areas: optional co-location of functions, where SUCI is calculated in UE, storage and handling of keys, priority of crypto algorithms, conditions for primary reauthentication and frequency of key setting, UICC properties, logging of certain events. More specific examples of these are given in Annex A.

- Network Slice-Specific Authentication and Authorization (NSSAA): Additional authentication and authorization that is performed beyond primary authentication is based on an operator or enterprise's risk associated with access to a dedicated slice. Different slices may have different associated risks and therefore while access to a dedicated slice may require the services of NSSAA Function (NSSAAF), while, access to a different dedicated slice with a different risk profile may not need the additional authentication services provided by the NSSAA. Evaluate risks associated with specific dedicated slices and provide guidance accordingly on the use of NSSAA by operators and enterprises.

### Recommendations to Industry

#### Previous CSRIC Recommendations

CSRIC VII recommends that industry rely upon CSRIC recommendations to mitigate threats to the 5G SA system, specifically CSRIC VI, V, and IV Reports.

#### NAS Signaling Confidentiality

3GPP TS 33.501 specifies mandatory (e.g., requires vendor implementation) support for protection of the NAS signaling confidentiality, but optional for service providers to use.

Given this standards requirement, CSRIC VII recommends only non-user identity related information shall be conveyed prior to security context is established.

Note, after security context is established all NAS messages are encrypted according to 3GPP TS 33.501.

#### User Plane Confidentiality

3GPP TS 33.501 specifies mandatory (e.g., requires vendor implementation) support for protection of the User plane confidentiality, but optional for service providers to use.

Given this standards requirement, CSRIC VII recommends User plane confidentiality protection over the access stratum be done at PDCP layer.

Confidentiality protection for UP is applied at the PDCP layer, and no layers below PDCP are confidentiality protected. User data sent via UPF may be confidentiality protected.

#### User Plane Integrity

3GPP TS 33.501 specifies mandatory UE support of integrity protection and replay protection of user data between the UE and the gNB, but the data rates at which it is supported is different between Release 15 and 16, and it is optional for service providers to use.

CSRIC VII recommends that device OEM and network infrastructure vendors support the Release 16 full rate capability, along with 128-NIA3 as defined in Annex D of 3GPP TS 33.501, and for operators to implement according to the service requirement.

CSRIC VII recommends that user data integrity is mandatory for Release 16 U.S. deployments.

While the goal is for mandatory user data integrity in Release 16 U.S. deployment, CSRIC VII recognizes that during operator network transitions to consistent and ubiquitous 5G SA availability and coverage, operators may defer deploying user plane data integrity protection during this transition period.  Examples of impacts to providing seamless integrity protection include:

- Significant user base of Release 15 UEs not supporting user plane integrity protection at full rate

- 4G/LTE overlay networks unable to support user plane integrity protection

#### RRC Signaling Confidentiality

3GPP TS 33.501 specifies mandatory (e.g., requires vendor implementation) support for protection of the RRC signaling confidentiality, but optional for service providers to use.

Given this standards requirement, CSRIC VII recommends protection of the RRC-signaling confidentiality. Only non-identity related information shall be conveyed prior to security context is established.

#### Subscription Permanent Identifier/ International Mobile Subscriber Identity (SUPI/IMSI) Privacy

3GPP TS 33.501 specifies mandatory (e.g., requires vendor implementation) support for protection of the SUPI/IMSI privacy, however 3GPP allows for some exceptions where the Subscription Concealed Identifier (SUCI) may use null scheme (i.e., the identity is not protected).

CSRIC VII recommends that devices and networks in the U.S. use IMSI privacy (SUCI), and do not use null encryption scheme except when the UE is requesting emergency services.

It is recommended that no other exceptions allowed by 3GPP in Release 16 (for null encryption scheme SUCI) be used by devices or networks in the U.S.  This may result in roaming 5G devices configured by operators from outside the U.S being unable to connect to 5G SA (option 2) networks. They can use 4G LTE networks instead.

#### Network Security – IPSec

3GPP TS 33.501 specifies mandatory (e.g., requires vendor implementation) support of protection of the network security – IPSec, but optional for service providers to use.

CSRIC VII recommends the use of IPSec or use of a tunneling technology for transport (e.g., VPN tunnels) for protection of network security.

#### Core Network Security – Transport Layer Security (TLS)

3GPP 33.501 specifies mandatory (e.g., requires vendor implementation) support of protection of the core network security – TLS, but optional for service providers to use.

CSRIC VII recommends the use of TLS for SBA interfaces and for non-SBA use of a tunneling technology for transport (e.g., VPN tunnels) for protection of core network security.

# Annex A: Examples of Other Findings and Limitations

This Annex lists some examples where, as mentioned in Section 6.1.5, during the course of the analysis, it was determined that some 3GPP TS 33.501 specification requirements with optional choices fall outside the question of optional-to-use security features that are the primary consideration for this study. These items were consequently left without recommendations. Examples listed by categories:

- Location of services

  - Example, Section 5.9.2.4: “The SCP has interfaces with Network Functions (NF) and peer SCPs within the PLMN. The interface between the SCP and the NFs and between the two SCPs shall fulfil the following requirements:
- Mutual authentication shall be performed between the SCP and NFs, and between the two SCPs within the PLMN.
- All communication between the SCP and NFs and between SCPs shall be confidentiality, integrity and replay protected.
If SCP endpoints are co-located with the NFs, the above two requirements may be satisfied by colocation”

- Options on where SUCI is calculated in UE

  - Example, Section 5.2.5: “Based on home operator's decision, indicated by the USIM, the calculation of the SUCI shall be performed either by the USIM or by the ME.”

- Storage and handling of keys

  - Example, Section 6.1.1.1: “The KAUSF may be securely stored in the AUSF based on the home operator's policy on using such key.”

  - Example, Section 6.1.3.2.0: “3. The AUSF shall store the XRES* temporarily together with the received SUCI or SUPI. The AUSF may store the KAUSF.”

  - Example, Section 6.2.2.1: “In case EAP-AKA' is used as authentication method, the AUSF shall derive a key KAUSF from CK' and IK' for EAP-AKA' as specified in clause 6.1.3.1. The KAUSF may be stored in the AUSF between two subsequent authentication and key agreement procedures.”

  - Example, Section 6.2.2.1: “the AMF shall generate NH and transfer it to the gNB, together with the corresponding NCC value. 
The AMF may also transfer an NH key, together with the corresponding NCC value, to another AMF, cf. clause 6.9.”

  - Example, Section 6.2.2.2: “Storage of the KAUSF at the UE is optional. If the USIM supports 5G parameters storage, KAUSF shall be stored in the USIM. Otherwise, KAUSF shall be stored in the non-volatile memory of the ME.”

- Priority of crypto algorithms

  - Example, Section 6.7.1.1: “Each AMF shall be configured via network management with lists of algorithms which are allowed for usage. There shall be one list for NAS integrity algorithms, and one for NAS ciphering algorithms. These lists shall be ordered according to a priority decided by the operator.”

  - Example, Section 6.7.1.1: “Each AMF shall be configured via network management with lists of algorithms which are allowed for usage. There shall be one list for NAS integrity algorithms, and one for NAS ciphering algorithms. These lists shall be ordered according to a priority decided by the operator.”

- Conditions for primary reauthentication

  - Example, Section 6.3.2.2: “When the UE is registered in the same AMF in the same PLMN serving network over both 3GPP and non-3GPP accesses, the UE shall establish two NAS connections with the network. Upon receiving the registration request message, the AMF should check whether the UE is authenticated by the network. The AMF may decide to skip a new authentication run in case there is an available 5G security context for this UE by means of 5G-GUTI, e.g. when the UE successfully registered to 3GPP access, if the UE registers to the same AMF via non-3GPP access, the AMF can decide not to run a new authentication if it has an available security context to use. In this case, the UE shall directly take into use the available common 5G NAS security context and use it to protect the registration over the non-3GPP access.”

- Frequency of key setting

  - Example, Section 6.2.3.1: “Key setting happens at the end of successful authentication procedure. Authentication and key setting may be initiated by the network as often as the network operator wishes when an active NAS connection exists. Key setting can occur as soon as the identity of the mobile subscriber (i.e., 5G-GUTI or SUPI) is known by the AMF.”

- UICC properties

  - Example, Section 6.1.1.1: “The USIM shall reside on a UICC. The UICC may be removable or non removable.”

- OAM / logging of events?

  - Example, Section 6.6.1: “Additionally, the SMF may log the event and may take additional measures, such as raising an alarm.”

# Annex B: Glossary of Terms

3GPP	3rd Generation Partnership Project

4G 	Fourth Generation

5G	Fifth generation

5GC	5G core

5G NR	5G New Radio

5GPPP	5G Infrastructure Public Private Partnership

AF	Application function

AKA	Authentication and Key Agreement

AMF	Access and mobility function

AN	Access Network

API	Application Programming Interface

ARIB 	Association of Radio Industries and Businesses

AS	Access Stratum

ATIS	Alliance for Telecommunications Industry Solutions

AUSF	Authentication server function

BGP	Border gateway protocol

BIS	Bureau of Industry and Security

BITAG 	Broadband Internet Technical Advisory Group

BSS	Base station subsystem

BWP	Bandwidth part

CATLs 	CTIA Authorized Test Labs

CCSA	China Communications Standards Association

CIoT	Cellular Internet of Things

CMMC 	Cybersecurity Maturity Model Cybersecurity

COVID	Coronavirus Disease

CP	Control Plane

CR 	Change Request

C-RNTI	Cellular Radio Network Temporary Identifier

CRS	Cell-specific reference signal

CSA	Cloud Security Alliance

CSCC	Communications Sector Coordinating Council

CSDE	Council to Secure the Digital Economy

CSRIC	Communications Security, Reliability and Interoperability Council

CTIA 	CTIA – The Wireless Association

CU 	Central unit

DDoS	Distributed denial of service

DL	Downlink

DHS	Department of Homeland Security

DoC	Department of Commerce

DoD	Department of Defense

DU	Distributed units

EAP	Extensible Authentication Protocol

EAR	Export Administration Regulation

EARFCN	EUTRA Absolute Radio Frequency Channel Number

eCAPIF	Enhancements for Common API Framework for 3GPP Northbound APIs

EIR	Equipment Identity Register

eMBB	Enhanced mobile broadband

eMIMO	Enhanced Multiple Input Multiple Output

eNB	Evolved NodeB

EPC	Evolved Packet Core

EPS	Evolved Packet System

ESP	Encapsulating Security Payload

ETSI	European Telecommunications Standards Institute

EUTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FCC	Federal Communications Commission

gNB	generation Node B

GETS	Government Emergency Telecommunications Service

GDPR	General Data Protection Regulation

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Services

GSMA	Global System for Mobile Communications Association

HARQ	Hybrid ARQ

HLR	Home Location Register

HO 	Handover

HSS	Home Subscriber System

HTTP	Hypertext Transfer Protocol

ICT	Information and Communications Technology

IEC	International Electrotechnical Commission

IEEE	Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

IMSI	International Mobile Subscriber Identity

IMT-2020	International Mobile Telecommunications

IOPS	Isolated E-UTRAN Operation for Public Safety

IoT 	Internet of Things

IP	Internet protocol

IPSec	Internet Protocol Security

ISO	International Organization for Standardization

ISP	Internet service providers

IT 	Information technology

ITU	International Telecommunication Union

ITU-R 	ITU Radiocommunication Sector

KeNB	eNodeB Key, in use at cell site

KeNB*	eNodeB Key, used to calculate at target cell site

KNASint	Non-Access Stratum Integrity Protection Key

KPI	Key performance indicator

LPWA	Low power wide area

LTE 	Long-term evolution

LSB	Least Significant Byte

MAC-I	Message Authentication Code

MACSec	Message Authentication Code Security

MCC	Mobile Country Code

MEC	Mobile edge computing

MIB	Master information block

MNC	Mobile Network Code

MME	Mobile management entity

mMTC	Massive machine-type communications

mmWave 	millimeter wave

MPS	Multimedia Priority Service

MSC	Mobile switching center

NAS	Non-Access Stratum

NAS-MAC	Non-Access Stratum Message Authentication Code

NAT-T	Network Address Translation Traversal

NB-IoT	Narrowband Internet of Things

NCC	Next Hop Chaining Counter

NEF	Network exposure function

NF	Network functions

NFV	Network function virtualization

NGC	Next generation core

NGMN	Next Generation Mobile Network

NG-RAN	Next generation radio access network

NH	Next Hop

NIST	National Institute of Standards and Technology

NR	New Radio

NPRM	Notice of Proposed Rulemaking

NR	New radio

NRF	Network resource function

NSA	Non-standalone

NSSAI	Network Slice Selection Assistance Information

NSSF	Network slice selection function

OTA	Online Trust Alliance

PBCH 	Physical broadcast channel

PCF	Policy control function

PCFICH 	Physical control format indicator channel

PCI	Physical cell identity

PCRF	Policy and charging rules function

PGW	Packet gateway

PLMN	Public Land Mobile Network

PRBS	Physical resource blocks

PSA	Platform Security Architecture Joint Stakeholder Agreement

PSS	Primary synchronization signal

QBL	Qualified bidder lists

QML	Qualified manufacturer lists

RAN	Radio access network

RLF	Radio Link Failure

RRC	Radio resource control

SA	Standalone

SA3 	Security working group

SAE	System Architecture Evolution

SBA	Service-based architecture

SCP	Service communication proxy

SCAS 	Security Assurance Specifications

SCRM	Supply chain risk management

SD	Slice differentiator

SDN	Software-defined networking

SEAF	Security Anchor Function

SEG	Security Gateway

SEPP	Security Edge Protection Proxy

SGSN	Serving GPRS support node

SGW	Serving gateway

SIB	System information blocks

SMF	Session management function

S-NSSAI	Single Network Slice Selection Assistance Information

SS7	Signaling system 7

SSB	Synchronization signal block

SSS	Secondary synchronization signal

SST	Slice/Service type

S-TMSI	SAE-Temporary Mobile Subscriber Identity

SUPI 	Subscription Permanent Identifier

TAU	Tracking Area Update

TLS	Transport Layer Security

TS	Technical Specification

TSDSI	Telecommunications Standards Development Society, India

TTA	Telecommunication Technology Association (Korea)

TTC	Telecommunication Technology Association (Japan)

UDM	Unified data management

UE	User equipment

UL	Up Link

UMTS	Universal Mobile Telecommunications System

UPF	User plane function

URLLC	Ultra-reliable low-latency communication

URI	Uniform Resource Identifier

VLR	Visitor location register

WG	Working Group

WPS	Wireless Priority Service

Published Versions available at: https://www.3gpp.org/DynaReport/23501.htm

Published Versions available at: https://www.3gpp.org/DynaReport/22011.htm

Published Versions available at: https://www.3gpp.org/DynaReport/33501.htm

Published Versions available at: https://www.3gpp.org/DynaReport/33310.htm

Published Versions available at: https://www.3gpp.org/DynaReport/33210.htm

Available at: https://www.fcc.gov/files/csric6wg3finalreport32018pdf

Available at: https://www.fcc.gov/files/csric7reportriskintroducedby3gpppdf

https://csrc.nist.gov/publications/detail/sp/800-39/final

Section 5.2.3: “The UE shall support integrity protection of user data at any data rate, up to and including, the highest data rate supported by the UE.”

Section 5.13: The Network slice specific authentication and authorization function (NSSAAF) shall handle the Network Slice Specific Authentication requests from the serving AMF.

The NSSAAF is responsible to send the NSSAA requests to the appropriate AAA-S.

The NSSAAF shall support AAA-S triggered Network Slice-Specific Re-authentication and Re-authorization and Slice-Specific Authorization Revocation and translate any AAA protocol into a Service Based format.

NSSAAF shall translate the Service based messages from the serving AMF to AAA protocols towards AAA-P/AAA-S.

Section 14.4: Services provided by NSSAAF; Network Slice-Specific Re-authentication and Re-authorization procedure; AAA Server triggered Slice-Specific Authorization Revocation

Section 16.2: Authorization for network slice access: An authorized S-NSSAI (i.e. allowed S-NSSAI) shall be granted to a UE only after the UE has completed successfully primary authentication. At the end of the primary authentication, the AMF and UE may receive a list of allowed S-NSSAI, which the UE is authorized to access.

For certain S-NSSAIs, additional Network Slice Specific Authentication and Authorization (NSSAA) is required.

Section 16.3: Network slice specific authentication: This clause specifies the optional-to-use Network slice-specific authentication and authorization between a UE and a AAA server (AAA-S) which may be owned by an external 3rd party enterprise. Network slice-specific authentication and authorization uses a User ID and credentials, different from the 3GPP subscription credentials (e.g., SUPI and credentials used for PLMN access) and takes place after the primary authentication.

The EAP framework specified in RFC 3748 shall be used for Network slice-specific authentication and authorization between the UE and the AAA server. See section 6.1.8.1 for more detail.

Include Authentication

Service Based Interfaces (SBI); 13.1.1 TLS protection between NF and SEPP; 13.1.2 Protection between SEPPs. 13.3.1 Authentication and authorization between network functions and the NRF; 13.3.2 Authentication and authorization between network functions;13.3.8 Client credentials assertion and authentication

Optional approaches for deployment

Section 13.1.1.0: NOTE: Whether the SEPP and NFs within the SEPP’s PLMN use telescopic FQDN or the custom HTTP header is based on PLMN operator’s policy.

Section 13.3.2.3: When an NF receives message from other unauthenticated NF, the NF shall support error handling, and may send back an error message.

Service Based Interfaces (SBI); ;13.3.5 Authentication between SEPP and SCP; 13.3.6 Authentication and authorization between SCP and network functions; 13.3.7 Authentication and authorization between SCPs

Section 9.9: Non-SBA interfaces internal to the 5G Core such as N4 and N9 can be used to transport signalling data as well as privacy sensitive material, such as user and subscription data, or other parameters, such as security keys. Therefore, these interfaces shall be confidentiality, integrity, and replay protected.

Roaming interfaces between PLMNs except for N32, shall be confidentiality, integrity, and replay protected. Protection for the N32 interface is specified in clauses 13.1 and 13.2.

For the protection of the above mentioned internal and roaming interfaces except N32, NDS/IP shall be used as specified, unless security is provided by other means, e.g. physical security. A SEG may be used to terminate the NDS/IP IPsec tunnels.

Section 9.9 A SEG may be used to terminate the NDS/IP IPsec tunnels.

Section 5.9.3.4, The IPUPS shall only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others. The IPUPS shall discard malformed GTP-U messages

Section 9.8.2. The security mechanisms for the F1 interface connecting the IAB-node to the IAB-donor-CU are detailed in clause M.3.3 of this document. NOTE 4: The support of DTLS for F1-C, between the IAB-node (gNB-DU) and the IAB-donor-CU, is optional for the IAB-node and the IAB-donor-CU.

Section 5.12: Requirements on 5G-RG: The 5G-RG shall be equipped with UICC where the subscription credentials resides. If provisioned by the home operator, the 5G-RG shall store the Home Network Public Key required for concealing the SUPI in the UICC.

The 5G-RG shall support all the security requirements and features of the UE defined in clause 5.2 (includes User data and signalling data confidentiality; User data and signalling data integrity, and Subscriber privacy). See top 8 security categories of this table.

Section 6.1.1: NOTE A: For standalone non-public networks when an authentication method other than 5G AKA or EAP-AKA' is used, Annex I.2 applies. NOTE 5: For non-public network (NPN) security the Annex I of the present document provides details.

Section 6.1.1.4.1: The SN Id identifies the serving PLMN and, except for standalone non-public networks, is defined as SNN-network-identifier in TS 24.501. NOTE 1: For standalone non-public networks, the definition of SN Id is given in Annex I.3

Section 6.2.1: NOTE 1: The key hierarchy for standalone non-public networks when an authentication method other than 5G AKA or EAP-AKA' is used is given in Annex I.2.3.

I.3: Serving network name for standalone non-public networks

9.10 Security mechanisms for the interface between W-5GAN and 5GC. The security of the N2 interface between the W-5GAN and the AMF is as defined in clause 9.2 of the present document.

The security of the N3 interface between the W-5GAN and the UPF is as defined in clause 9.3 of the present document.

On the W-AGF side a SEG may be used to terminate the IPsec tunnels. Add 7B: Security for wireline access to the 5G core network

N2 interface is between gNB and AMF.

N3 interface is between gNB and UPF.

Under “Annex B (informative): Using additional EAP methods for primary authentication” and “B.2.2 Revocation of subscriber certificates”. It is provided as an example on how the 5G authentication framework for primary authentication can be applied to EAP methods other than  EAP-AKA' The additional EAP methods are only intended for private networks or with IoT devices in isolated deployment scenarios, i.e. roaming is not considered, as specified in TS 22.261. When the 5G system is deployed in private networks, the SUPI and SUCI should be encoded using the NAI format as specified in TS 23.501.

More information about GSMA's NESAS program can be found at https://www.gsma.com/security/network-equipment-security-assurance-scheme/

See https://www.fcc.gov/document/fcc-seeks-comment-implementation-diameter-best-practices

See  https://transition.fcc.gov/Daily_Releases/Daily_Business/2017/db0824/DA-17-799A1.pdf

See CSRIC VI Report on Best Practices and Recommendations to Mitigate Security Risks to Emerging 5G Wireless Networks (September 2018)

See CSRIC V Report on Legacy Systems Risk Reductions (March 2017). https://www.fcc.gov/files/csric5-wg10-finalreport031517pdf

See CSRIC VI Report on Recommendations to Mitigate Security Risks for Diameter Networks (March 2018)  https://www.fcc.gov/file/13925/download

This wording is to be interpreted as the KAUSF is required to be securely stored, but the operator may optionally securely store the KAUSF in the AUSF.