| 29 MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS June 26 to 29, 2017 Orlando, Florida, United States of America |  | OEA/Ser.L/XVII.4.2.29 CCP.II-RADIO/doc. 4xxx x June 2017 Original: English |  |
| --- | --- | --- | --- |
|  | U.S. PRELIMINARY VIEW ON WRC-19 AGENDA ITEM 1.15 |  |  |
|  | (Items on the Agenda: 3.1) |  |  |
|  | (Document submitted by the delegation of the United States of America) |  |  |

Introduction

This document contains an attachment including the USA preliminary view on WRC-19 Agenda Item 1.15 for consideration in CITEL’s preparation to WRC-19 Agenda Item 1.15.

Attachment

Agenda Item 1.15:  to consider identification of frequency bands for use by administrations for the land-mobile and fixed services applications operating in the frequency range 275-450 GHz, in accordance with Resolution 767 (WRC-15).

BACKGROUND:  At present, there are no international allocations for radiocommunications services above 275 GHz in the Radio Regulations (RR’s).  However, footnote No. 5.565 does make identifications for radio astronomy, earth exploration-satellite (passive) and space research (passive) services.  Recent advances in microwave technology make possible the use of this spectrum by active services for communications and related uses.  Consistent with No. 5.565, frequencies for fixed and land mobile use could be utilized above 275 GHz, provided “all practicable steps” are taken to protect passive services.

Report ITU-R RA.2189 “Sharing between the radio astronomy service and active services in the frequency range 275-3 000 GHz” indicates that the radio astronomy service can share with terrestrial systems due to propagation conditions and power limitations of current active services technologies.  The space research service (passive) and the Earth exploration-satellite service (passive) may also be able to share frequencies with the active services; however, studies are needed to demonstrate this.

PRELIMINARY VIEW:

USA

The United States is of the view that it may be possible to develop a similar footnote to that in No. 5.565 for land-mobile and fixed services, identifying bands for terrestrial active service use.  To this end, the United States supports studies in the ITU-R on sharing and compatibility between passive and active services as well as spectrum needs for the land-mobile and fixed services for WRC-19 agenda item 1.15 under the terms of Resolution 767 (WRC-15).

__________