| 29 MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS June 26 to 29, 2017 Orlando, Florida, United States of America |  | OEA/Ser.L/XVII.4.2.29 CCP.II-RADIO/doc. 4xxx x June 2017 Original: English |  |
| --- | --- | --- | --- |
|  | U.S. PRELIMINARY VIEW ON WRC-19 AGENDA ITEM 1.9.2 |  |  |
|  | (Items on the Agenda: 3.1) |  |  |
|  | (Document submitted by the delegation of the United States of America) |  |  |

Introduction

This document contains an attachment including the USA preliminary view on WRC-19 Agenda Item 1.9.2 for consideration in CITEL’s preparation to WRC-19 Agenda Item 1.9.2.

Attachment

Agenda Item 1.9.2: modifications of the Radio Regulations, including new spectrum allocations to the maritime mobile-satellite service (Earth-to-space and space-to-Earth), preferably within the frequency bands 156.0125-157.4375 MHz and 160.6125-162.0375 MHz of Appendix 18, to enable new VHF data exchange (VDES) satellite component, while ensuring that this component will not degrade the current terrestrial components, applications specific messages (ASM) and AIS operations and not impose any additional constraints on existing services in these and adjacent frequency bands as stated in recognizing d) and e) of Resolution 360 (Rev. WRC-15).

BACKGROUND: RESOLUTION 360 (REV. WRC-15) “Consideration of regulatory provisions and spectrum allocations to the maritime mobile-satellite service to enable the satellite component of the VHF Data Exchange System and enhanced maritime radiocommunications”, invites ITU-R to conduct, as a matter of urgency, and in time for WRC-19, sharing and compatibility studies between VDES satellite components and incumbent services in the same and adjacent frequency bands specified in recognizing d) and e) to determine potential regulatory actions, including spectrum allocations to the MMSS (Earth-to-space and space-to-Earth) for VDES applications. To this end, the ITU-R has initiated sharing studies between the proposed VDES satellite (VDE-SAT) frequencies and the incumbent services in the same and adjacent bands so that  this component does not impose any additional constraints on existing services in these and adjacent frequency bands as stated in recognizing d) and e) of Resolution 360 (Rev. WRC-15). The United States believes that the satellite component of the VDES could be beneficial towards enhancing maritime navigation and safety related applications on a global basis.

PRELIMINARY VIEW:

USA

The United States supports the ITU-R studies prescribed in Resolution 360 (Rev. WRC-15) and these studies should also take into account the protection of existing terrestrial services which operate in these and adjacent frequency bands.

__________________________