| 32 MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS December 3 to 7, 2018 Brasilia, D.F. Brazil |  | OEA/Ser.L/XVII.4.2.32 CCP.II-RADIO/doc. [ ]/18 [ ] October 2018 Original: English |  |
| --- | --- | --- | --- |
|  | U.S. PROPOSAL ON WRC-19 AGENDA ITEM 1.16 |  |  |
|  | (Item on the Agenda: 3.1) |  |  |
|  | (Document submitted by the delegation of the United States of America) |  |  |

Introduction

This document contains an attachment including the USA proposal on WRC-19 Agenda Item 1.16 (5250 – 5925 MHz) for consideration in CITEL’s preparation to WRC-19 Agenda Item 1.16.

Attachment

PROPOSALS FOR THE WORK OF THE CONFERENCE

Agenda Item 1.16:  To consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 (WRC-15).

Background Information:  Resolution 239 (WRC-15) calls for the ITU-R to study WAS/RLAN technical characteristics and operational requirements in the 5 GHz frequency range.  The resolution also calls for the ITU-R to conduct studies with a view to identify potential WAS/RLAN mitigation techniques to facilitate sharing with incumbent systems in the frequency bands 5 150-5 350 MHz, 5 350-5 470 MHz, 5 725-5 850 MHz and 5 850-5 925 MHz, while ensuring the protection of incumbent services including their current and planned use.

Recognizing results of the ITU-R studies and existing uses in the frequency bands between 5 250 MHz and 5 925 MHz, this proposal is for NOC for the 5 250-5 350 MHz, 5 350-5 470 MHz, 5 725-5 850 MHz and 5 850-5 925 MHz bands.  Countries wishing to implement WAS/RLANs in the frequency should take into account RR No. 5.453.

Proposal:

Article 5

Frequency Allocations

Section IV – Table of Frequency Allocations

(See No. 2.1)

NOC			USA/1.16/1

| 5 250-5 570 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH 5.447D 5.447E 5.448 5.448A |  |  |
| 5 255-5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH (active) 5.447E 5.448 5.448A |  |  |
| … |  |  |

Reasons:  Since the adoption of Resolution 229 (Rev. WRC-12) at WRC-03, millions of WAS/RLAN (such as Wi-Fi) devices have been deployed in the band 5 250-5 350 MHz.  Studies in response to invite c of Resolution 229 have shown that changing the WAS/RLAN operating conditions in the band 5 250-5 350 MHz would not ensure protection of incumbent radiodetermination services and EESS (active) sensors.

NOC			USA/1.16/2

| 5 250-5 570 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B RADIOLOCATION 5.448D AERONAUTICAL RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448C |  |  |
| 5 460-5 470 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION 5.448D RADIONAVIGATION 5.449 SPACE RESEARCH (active) 5.448B |  |  |
| … |  |  |

Reasons:  Previous ITU-R studies show that sharing between WAS/RLAN operating in the band 5 350-5 470 MHz and the EESS (active) systems or between WAS/RLAN and different radar systems would not be feasible unless WAS/RLAN implement additional mitigation measures.  After further review of currently available mitigation measures, study results show that there are no feasible mitigation techniques to facilitate sharing between WAS/RLAN and EESS (active) or between WAS/RLAN and radar systems in the 5 350-5 470 MHz band.

NOC			USA/1.16/3

| 5 570-6 700 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur | 5 725-5 830 RADIOLOCATION Amateur |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  |
| 5.150 5.451 5.453 5.455 | 5.150 5.453 5.455 |  |
| … |  |  |

Reasons:  WAS/RLAN already operate in various countries throughout the world within the frequency band 5 725-5 850 MHz.  In Region 2, the band 5 725-5 825 MHz is also used by WAS/RLAN.

NOC			USA/1.16/4

| 5 570-6 700 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation |
| 5.150 | 5.150 | 5.150 |
| … |  |  |

Reasons: The mobile service is co-primary in the 5 850-5 925 MHz band and no WRC action is necessary.  Various countries have already implemented RLAN as well as other mobile applications under the mobile allocation in this band.

____________________