| 34 MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS August 12 to 16, 2019 Ottawa, Canada |  | OEA/Ser.L/XVII.4.2.33 CCP.II-RADIO/doc. 4954/19 rev 17 July 2019 Original: English |  |
| --- | --- | --- | --- |
|  | U.S. PROPOSAL ON WRC-19 AGENDA ITEM 1.9.1 |  |  |
|  | (Item on the Agenda: 3.1 (SGT2-A)) |  |  |
|  | (Document submitted by the delegation of the United States of America) |  |  |

Introduction

This document contains an attachment including the updated USA proposal on WRC-19 Agenda Item 1.9.1 for consideration in CITEL’s preparation to WRC-19 Agenda Item 1.9.1.

ATTACHMENT

Agenda Item 1.9.1	:   regulatory actions within the frequency band 156-162.05 MHz for autonomous maritime radio devices to protect the GMDSS and automatic identifications system (AIS), in accordance with Resolution 362 (WRC-15);

Background

The purpose of this agenda item is to consider recognition of autonomous maritime radio devices (AMRD) that can enhance safety of navigation while ensuring that these AMRDs do not harm the integrity of the global maritime distress and safety system (GMDSS) and the automatic identification system (AIS).

Studies on this WRC-19 agenda item are based on the following definition of AMRD:

An AMRD is a mobile station; operating at sea and transmitting independently of a ship station or a coast station. Two groups of AMRD are identified:

–	Group A: AMRD that enhance the safety of navigation;

–	Group B: AMRD that do not enhance the safety of navigation (AMRD which deliver signals or information which do not concern the vessel can distract or mislead the navigator and degrade the safety of navigation).

AMRD Group A are devices that enhance the safety of navigation may be subject to IMO SOLAS regulations for the presentation of information to the navigators on board vessels. Other AMRD may be considered as Group B AMRD depending on certain characteristics and parameters.

The relevant characteristics for the operation of AMRD are contained in the most recent version of Recommendations ITU-R M.493, and ITU-R M.1371. Group A, AMRD that enhance the safety of navigation, should use the frequencies of the current RR Appendix 18 for digital selective calling (channel 70 (156.525 MHz)), and the AIS channels (channels AIS 1 (161.975 MHz) and AIS 2 (162.025 MHz)).

Group B, AMRD that do not enhance the safety of navigation, which are also operating in the maritime environment should only use channel 2006 (160.90 MHz) of Appendix 18.

Proposals

MOD	USA/1.9.1/1

APPENDIX 18 (REV.WRC-19)

Table of transmitting frequencies in the
VHF maritime mobile band

…

Specific notes

…

f) 	The frequencies 156.300 MHz (channel 06), 156.525 MHz (channel 70), 156.800 MHz (channel 16), 161.975 MHz (AIS 1) and 162.025 MHz (AIS 2) may also be used by aircraft stations for the purpose of search and rescue operations and other safety-related communication. The frequencies 156.525 MHz (channel 70), 161.975 MHz (AIS 1) and 162.025 MHz (AIS 2) may also be used by autonomous maritime radio devices Group A for digital selective calling and/or AIS-technology  as described in the most recent versions of Recommendations ITU R M.493 and M.1371. (WRC-19)

…

r) 	Administrations may apply this frequency for the usage of autonomous maritime radio devices Group B using AIS-technology on a non-interference basis limited to transmitter e.i.r.p. of 100 mW, and a height of the antenna not exceeding 1 m above the surface of the sea. (WRC-19)

Reasons: These modifications to Appendix 18 allow the operation of AMRD AIS-technology based Group A, devices which have been identified to enhance the maritime safety of navigation; and AMRD Group B device that do not enhance maritime safety of navigation but operate in the maritime environment.

…

SUP	USA/1.9.1/2

RESOLUTION 362 (REV.WRC-15)

Autonomous maritime radio devices operating in 
the frequency band 156-162.05 MHz

Reasons:	It is proposed to suppress Resolution 362 (WRC-15) since the studies have been completed and the identification of frequencies in Appendix 18 for AMRD has been made by WRC-19.

_____________________________________________